Sinclair ZX Spectrum Emulator 'Z80' v3.04  -  10/8/96  -  by G.A. Lunter

5.  TECHNICAL INFORMATION

Contents:

5.1   The Spectrum 48K

5.2   The Spectrum 128K

5.3   The AY-3-8912 sound chip

5.4   The ZX Printer

5.5   The Interface I

5.6   The SamRam

5.7   The Multiface 128

5.8   The Z80 microprocessor

5.9   File formats

5.1  The Spectrum 48K

In this section, the hardware of the 48K Spectrum is discussed. In this

section, 'Spectrum' by itself refers to the 48K machine only.

The Spectrum is at the hardware level a very simple machine.  There's

the 16K ROM which occupies the lowest part of the address space, and

48K of RAM which fills up the rest.  An ULA which reads the lowest 6912

bytes of RAM to display the screen, and contains the logic for just one

I/O port completes the machine, from a software point of view at least.

Every even I/O address will address the ULA, but to avoid problems with

other I/O devices only port FE should be used.  If this port is written

to, bits have the following meaning:

Bit   7   6   5   4   3   2   1   0

�������������������������������Ŀ

�   �   �   � E � M �   Border  �

���������������������������������

The lowest three bits specify the border colour; a zero in bit 3

activates the MIC output, and a one in bit 4 activates the EAR output

(which sounds the internal speaker).  The real Spectrum also activates

the MIC when the ear is written to; the emulator doesn't.  This is no

problem; MIC is only used for saving, and when saving the Spectrum

never sounds the internal speaker.  The upper three bits are unused.

If port FE is read from, the highest eight address lines are important

too.  A zero on one of these lines selects a particular half-row of

five keys:

IN:    Reads keys (bit 0 to bit 4 inclusive, in that order)

#FEFE  SHIFT, Z, X, C, V            #EFFE  0, 9, 8, 7, 6

#FDFE  A, S, D, F, G                #DFFE  P, O, I, U, Y

#FBFE  Q, W, E, R, T                #BFFE  ENTER, L, K, J, H

#F7FE  1, 2, 3, 4, 5                #7FFE  SPACE, SYM SHFT, M, N,

A zero in one of the five lowest bits means that the corresponding key

is being pressed.  If more than one address line is made low, the

result is the logical AND of all single inputs, so a zero in a bit

means that at least one of the corresponding keys are pressed.  For

example, only if each of the five lowest bits of the result from

reading from port 00FE (for instance by XOR A/IN A,(FE)) is one, no key

is pressed.

A final remark about the keyboard.  It is connected in a matrix-like

fashion, with 8 rows of 5 columns, as is obvious from the above

remarks.  Any two keys pressed simultaneously can be uniquely decoded

by reading from the IN ports; however, if more than two keys are

pressed decoding may not be uniquely possible.  For instance, if you

press Caps shift, B and V, the Spectrum will think also the Space key

is pressed, and reacts by giving the 'Break into Program' report.  This

matrix behaviour is also emulated - without it, Zynaps for instance

won't pause when you press 5,6,7,8 and 0 simultaneously.

Bit 5 (value 64) of IN-port FE is the ear input bit.  When the line is

silent, its value is zero, except in the early Model 2 of the Spectrum,

where it was one.  When there is a signal, this bit toggles.  The

Spectrum loading software is not sensitive to the polarity of this bit

(which it definitely should not be, not only because of this model

difference, but also because one cannot be sure the tape recorder

does not change the polarity of the recorded signal!)  Some old

programs rely on the fact that bit 5 is always one (for instance

Spinads); for these programs the emulator can mimic a Model 2 Spectrum.

Bits 6 and 7 are always one.

The ULA with the lower 16K of RAM, and the processor with the upper 32K

RAM and 16K ROM are working independently of each other.  The data and

address buses of the Z80 and the ULA are connected by small resistors;

normally, these effectively decouple the buses.  However, if the Z80

wants to read of write the lower 16K, the ULA halts the processor if it

is busy reading, and after it's finished it lets the processor access

lower memory through the resistors.  A very fast, cheap and neat design

indeed!

If you run a program in the lower 16K of RAM, or read or write in that

memory, the processor is halted sometimes.  This part of memory is

therefore somewhat slower than the upper 32K block.  This is also the

reason that you cannot write a sound- or save-routine in lower memory;

the timing won't be exact, and the music will sound harsh.  Also,

INning from port FE will halt the processor, because the ULA has to

supply the result.  Therefore, INning from port FE is a tiny bit slower

on average than INning from other ports; whilst normally an IN A,(nn)

instruction would take 11 T states, it takes 12.15 T states on average

if nn=FE. See below for more exact information.

If the processor reads from a non-existing IN port, for instance FF,

the ULA won't stop, but nothing will put anything on the data bus.

Therefore, you'll read a mixture of FF's (idle bus), and screen and

ATTR data bytes (the latter being very scarce, by the way).  This will

only happen when the ULA is reading the screen memory, 61.5% (192/312)

of the 1/50th second time slice in which a frame is generated.  The

other 38.5% of the time the ULA is building the border or generating a

vertical retrace.  This behaviour is actually used in some programs,

for instance by Arkanoid, and Z80 also emulates this.

Finally, there is an interesting bug in the ULA which also has to do

with this split bus.  After each instruction fetch cycle of the

processor, the processor puts the I-R register 'pair' (not the 8 bit

internal Instruction Register, but the Interrupt and R registers) on

the address bus.  The lowest 7 bits, the R register, are used for

memory refresh.  However, the ULA gets confused if I is in the range

64-127, because it thinks the processor wants to read from lower 16K

ram very, very often.  The ULA can't cope with this read-frequency, and

regularly misses a screen byte.  Instead of the actual byte, the byte

previously read is used to build up the video signal.  The screen seems

to be filled with 'snow'; however, the Spectrum won't crash, and

program will continue to run normally.  There's one program I know of

that uses this to generate a nice effect: Vectron.  (which has very

nice music too by the way).  This effect has not been implemented

however - it's a bit useless (but maybe I'll include it in the future).

The processor has three interrupt modes, selected by the instructions

IM 0, IM 1 and IM 2.  In mode 1, the processor simply executes a RST

#38 instruction if an interrupt is requested.  This is the mode the

Spectrum is normally in.  The other mode that is commonly used is IM 2.

If an interrupt is requested, the processor first builds a 16 bit

address by combining the I register (as the high byte) with whatever

the interrupting device places on the data bus.  The processor then

fetches the 16-bit address at this interrupt table entry, and finally

CALLs the subroutine at that address.  Rodnay Zaks in his book

'Programming the Z80' states that only even bytes are allowed as low

index byte, but that isn't true.  The normal Spectrum contains no

hardware to place a byte on the bus, and the bus will therefore always

read FF (because the ULA also doesn't read the screen if it generates

an interrupt), so the resulting index address is 256*I+0FF.  However,

some not-so-neat hardware devices put things on the data bus when they

shouldn't, so later programs didn't assume the low index byte was 0FF.

These programs contain a 257 byte table of equal bytes starting at

256*I, and the interrupt routine is placed at an address that is a

multiple of 257.  A useful but not so much used trick is to make the

table contain FF's (or use the ROM for this) and put a byte 18 hex, the

opcode for JR, at FFFF.  The first byte of the ROM is a DI, F3 hex, so

the JR will jump to FFF4, where a long JP to the actual interrupt

routine is put.

In interrupt mode 0, the processor executes the instruction that the

interrupting device places on the data bus.  On a standard Spectrum

this will be the byte FF, coincidentally (...) the opcode for RST #38.

But for the same reasons as above, this is not really reliable.

The 50 Hz interrupt is synchronized with the video signal generation by

the ULA; both the interrupt and the video signal are generated by it.

Many programs use the interrupt to synchronize with the frame cycle.

Some use it to generate fantastic effects, such as full-screen

characters, full-screen horizon (Aquaplane) or pixel colour (Uridium

for instance).  Many modern programs use the fact that the screen is

'written' (or 'fired') to the CRT in a finite time to do as much

time-consuming screen calculations as possible without causing

character flickering:  although the ULA has started displaying the

screen for this frame already, the electron beam will for a moment not

'pass' this-or-that part of the screen so it's safe to change something

there. So the exact time in the 1/50 second time-slice at which the

screen is updated is very important.  Normally the emulator updates the

entire screen at once (50 times a second), and no best solution can be

given as to when exactly the screen should be updated.  The user can

select one of three possibilities (low, normal and high video

synchronisation, corresponding to a screen update after 1/200, 2/200 or

3/200 of a (relative) second after a Z80 interrupt) to try to get the

best results.  Try for instance Zynaps; with normal video

synchronisation the top four or five lines of the background move

out-of-phase with the rest, and your space-ship flickers in that

region.  With low video synchronisation the background moves smoothly

but the sprites flicker in all parts of the screen.  Only with high

video sync everything moves smoothly and doesn't flicker.

In Hi-resolution colour emulation mode, however, the emulator makes a

copy of every screen- and attribute-line in a buffer at the exact time

the ULA would display it.  Also, the exact times the border colour is

changed is stored.  Using this information the emulator builds the

screen; in this way, what you see on your PC monitor is exactly what a

real Spectrum would display on a television.  Remember Aquaplane, with

its full-width horizon?

Each line takes exactly 224 T states.  After an interrupt occurs, 64

line times pass before the byte 16384 is displayed.  At least the last

48 of these are actual border-lines.  I could not determine whether my

monitor didn't display the others or whether it was in vertical

retrace, but luckily that's not really important.  Then the 192

screen+border lines are displayed, followed by 56 border lines again.

This makes a total of 312 lines of 224 T states, or 69888 T states,

which is, at 3.5 MHz, very nearly 1/50th of a second.

Now for the timings of each line itself.  I define a screen line to

start with 256 screen pixels, then border, then horizontal retrace, and

then border again.  All this takes 224 T states.  Every half T state a

pixel is written to the CRT, so if the ULA is reading bytes it does so

each 4 T states (and then it reads two: a screen and an ATTR byte). The

border is 48 pixels wide at each side.  A video screen line is

therefore timed as follows: 128 T states of screen, 24 T states of

right border, 48 T states of horizontal retrace and 24 T states of left

border.

When an interrupt occurs, the running instruction has to be completed

first.  The Z80 samples the state of its interrupt request line at the

start of the last T state of each instruction.  The Z80 starts to act

upon an interrupt request at least 1, and at most 1+23 T states after

it is made active, as the slowest instructions (e.g. INC (IX+d), RL

(IX+d), EX (SP),IX) take 23 T states).  This difference, which may be

hard to control, is sometimes significant in practice for hi-resolution

colour effects.  When the Z80 is executing a HALT instruction, it is

effectively executing NOPs and not incrementing the PC until an

interrupt is received (and interrupts are enabled).  As a NOP takes 4

clocks to execute, in this case the start of the interrupt is fixed up

to 4 T states.

In all interrupt modes, the interrupt acknowledge cycle is basically an

ordinary M1 instruction fetch cycle with two wait states added, taking

up 5 T states.  Then, in mode 0, any instruction that is placed on the

bus in the acknowledge cycle is executed, whereas in mode 1 this is

always RST #38.  The Spectrum ordinarily leaves the bus floating at

interrupt time, so that in mode 0 also a RST #38 (opcode #FF) is

executed.  A RST #38 normally takes 11 T states, so that the complete

mode 0 or 1 (or 0/1) interrupt takes 13 T states.  (Thanks to Ian

Collier for correcting this in the FAQ version)

A mode 2 interrupt starts off with a 7 T state M1 cycle in which the

interrupt vector is read, followed by two stack-write cycles of 3 T

states each in which the program counter is pushed, and two memory-read

cycles of 3 T's each that read the interrupt address.  That is 19 T's.

Ian Collier states that an NMI cycle takes 15 T states.  My reference

(Mostek Technical Manual of the Z80) breaks it down as follows: a 5 T

M1 cycle in which the opcode is ignored, then two 3-T stack-write

cycles to push the program counter, which adds up to 11 T states.  I

haven't done any testing, so I cannot say anything definite here.

The ZX81 hardware generates a WAIT only 16 T states before it generates

an NMI, which, by some combined hardware and software wizardry,

generates one scanline on the television screen.  It seems therefore

that by executing a whole lot of slow instructions in a block, it is

possible to jam the horizontal synchonisation of the ZX81 video signal.

Has this ever been tried?

Now when to OUT to the border to change it at the place you want?

First of all, you cannot change the border within a 'byte', an 8-pixel

chunk. If we forget about the screen for a moment, if you OUT to port

FE after 14326 to 14329 T states (including the OUT) from the start of

the IM 2 interrupt routine, the border will change at exactly the

position of byte 16384 of the screen.  The other positions can be

computed by remembering that 8 pixels take 4 T states, and a line takes

224 T states.  You would think that OUTing after 14322 to 14325 T

states, the border would change at 8 pixels left of the upper left

corner of the screen.  This is right for 14322, 14323 and 14324 T

states, but if you wait 14325 T states the ULA happens to be reading

byte 16384 (or 22528, or both) and will halt the processor for a while,

thereby making you miss the 8 pixels.  This exception happens again

after 224 T states, and again after 448, an so forth.  These 192

exceptions left of the actual screen rectangle are the only ones;

similar things don't happen at the right edge because the ULA don't

need to read things there - it has just finished!

As noted above, reading or writing in low ram (or OUTing to the ULA!)

causes the ULA to halt the processor.  When and how much?  The

processor is halted each time you want to access the ULA or low memory

and the ULA is busy reading.  Of the 312 'lines' the ULA generates,

only 192 contain actual screen pixels, and the ULA will only read bytes

during 128 of the 224 T states of each screen line.  But if it does,

the processor seems to be halted for 64 T states.  It is not clear to

me when, and for how long exactly, the ULA halts the processor.

Sometimes the ULA even stops the processor when it is not interfering

with it (when it is busy making the border left or right of the screen

rectangle).

For 128K timings, which are slightly different, see the next section.

5.2  The Spectrum 128K

In comparison to the Spectrum 48K, and on the hardware side, the

Spectrum 128 offers more RAM (128K, you guessed it), more ROM (32K

instead of 16K), a soundchip, and a serial printer port.  Nothing

really spectacular.

Another difference with the 48K Spectrum is in the timing of the video

signals.  You can see this if you save something: the bars in the

border move differently.  The really important difference of the 128K

with respect to the video is that the 128K ULA is more relaxed in

giving the Z80 access to (screen) memory.  This allows programs to make

hi-resolution colour effects not only in the border, but also on the

screen itself.  Many 128K programs use this effect.  Note however that,

although the 128K ULA is more relaxed towards memory access, it does

still halt the Z80 occasionally.  Partly for this reason it is

impossible to have hi-res colour effect over the entire screen; there is

only time to change approximately half of it.  (This does not seem to

be true; the Shock Megademo manages to move 1-pixel thick lines up on

the screen one pixel per frame on the whole screen.  Very well done

indeed!  This is the only program I know to feature hi-res colour effect

over the entire width of the screen.  It might be, however, that this

program only changes the entire ATTR line every two scan line times.)

The basic video timings, with the Z80 out of the way, are as follows.

Each video line takes 228 T states, 4 T states more than on the 48K

Spectrum.  It starts with 128 T states of screen pixels (or border).

Then there's border, horizontal retrace and border again, of 100 T

states.  A complete '50 Hz' frame consists of 311 video lines (of which

a few are vertical retraces), that is, 1 less than for 48K models.  A

complete frame is 311 x 228 = 70908 T states long.

I don't know whether the 128K model uses a different crystal.  If not,

one frame on the Spectrum 128K is 1.5% longer than a 48K frame.

A quote from the +2 manual, page 279; Cliff Lawson writes: "For the

contended RAM [pages 4-7] (which shares time between the video

circuitry and the processor), during 128 out of every 228 CPU T states

(1 TV line), and during 192 out of every 311 TV lines (1 frame) the CPU

is allowed only 1 access to contended RAM in every 8 T states.  The CPU

is controlled by introducing wait states".  My guess is this holds true

for the 128K too.

Directly after an interrupt is generated by the ULA (so slightly before

the Z80 acts upon it), 63 video lines are written to the CRT.  A first

few may be verical retraces; this is difficult to find out without an

oscilloscope, but luckily we don't need this information.  Then 192

screen lines are written, and then 56 border lines and (possibly)

vertical retrace lines.  The first screen byte is written to the screen

14364 T states after the interrupt was generated.

Then the other things.  Memory is arranged in banks of 16K.  The bank

at 0000-3FFF contains either the original 48K ROM or the new 128K ROM.

The latter is active at reset.  The bank at 4000-7FFF always contains

RAM page 5.  The bank at 8000-BFFF is always page 2.  The bank at

C000-FFFF contains any page from 0-7, including page 2 and 5.  If page

2 or 5 is enabled in the high bank, every byte written in this bank is

mirrored in the other bank at 4000 or at 8000, and vice versa.  The

screen information is read, by the ULA, from the first 6912 bytes of

either page 5 or page 7.

All this is controlled by writing to port 7FFD (or, in fact, by writing

to any address with bit 15 and 1 zero; don't use this fact, though; it

does not work on the +3, where there's another port 1FFD, and neither

will this work in full generality on the emulator.)

Port #7FFD:

Bit    7   6   5   4   3   2   1   0

�������������������������������Ŀ

WRITE�   �   � P � R � S �  page no  �

���������������������������������

Bits 0-2 determine which page is to appear in bank C000-FFFF.  If S=0,

the ULA reads the screen from page 5, otherwise it reads page 7.  If

R=0, the 128K Rom is selected in bank 0000-3FFF; otherwise the 48K Rom.

If P=1, port #7FFD will be disabled and keep its value until the

computer is reset.  This bit is set if you select 48K Spectrum in the

128K startup menu, so that no 48K program is able to (accidentally)

swap itself into oblivion.

The sound chip of the Spectrum 128 is described in the next section.

5.3  The AY-3-8912 sound chip.

The following section was put together using information collected from

comp.sys.sinclair.  Thanks, and large parts of the information below,

are due to Alastair Booker, who put a detailed description on the net

in April '95, and to Ian Collier who recently carried out some thorough

investigations on the AY chip.

This chip is used in for instance the Sinclair ZX Spectrum 128/+2/+3,

Amstrad CPC 464/664/6128, Mattel Intellivision, Atari ST, Sega Master

System and the MSX.

The AY has 16 internal registers.  A register is selected by OUTing the

register number in bits 0-3 to port #FFFD (only A15, A14 and A1 are

decoded).  Then write to a register by OUTing to #BFFD, read it by

INning from #FFFD.  When reading from a register, unused bits are

always 0.  Reading always yields the value last written to the

register, except for R14 and R15 when bit 6 or 7 of R7 are reset (R14 /

R15 used for input).  On the AY-3-8912, when R7 bit 7 is reset, R15

always reads 255.  Writing to R14 or R15 when they are selected for

input does load the output register.

Here are the names of the AY registers:

Register        Name                                Bits used:

R0              Fine tone control (FTC) channel A      0-7

R1              Coarse tone control (CTC) channel A    0-3

R2              FTC channel B                          0-7

R3              CTC channel B                          0-3

R4              FTC channel C                          0-7

R5              CTC channel C                          0-3

R6              Noise generator pitch control          0-4

R7              Mixer and I/O control                  0-7

R8              Amplitude channel A                    0-4

R9              Amplitude channel B                    0-4

R10             Amplitude channel C                    0-4

R11             Envelope fine period control           0-7

R12             Envelope coarse period control         0-7

R13             Envelope control                       0-3

R14             RS232 i/o                              0-7

R15             I/O port 2                             0-7

The AY chip consists of three tone generators, one noise generator, an

envelope generator, three mixers, and three volume generators.

Tone generator A is controlled by R0 and R1.  It contains a 12 bit up

counter which is reset to 0 whenever it is larger than or equal to the

value of R1R0 (most significant bits are in R1), and is counted up at a

frequency of 221660 Hz (which is the driving frequency of the chip

divided by 8). Loading R0 or R1 takes effect directly.  If the internal

counter is below the new value, it simply continues; if it is above, it

immediately resets to 0.  Every time the internal counter is reset, the

tone generator changes it output from 0 to 1 or vice versa, so the

frequency of the tone generated is 110830/R1R0 Hz (one period consists

of two transitions). If R1R0 contains 0, the counter behaves as if R1R0

contained 1.

The noise generator contains a 5 bit up counter, which is reset to 0

whenever it is >= R6.  It is counted up at a frequency of 110830 Hz

(driving frequency divided by 16) [1].  Every time it reaches zero, it

randomly chooses 0 or 1 as its new output [2].  When R6 is zero, the

noise generated is the same as when R6 is 1.  Changes to R6 take effect

only when the internal counter reaches 0.

Bit      7    6    5    4    3    2    1    0

���������������������������������������Ŀ

R7    � D2 � D1 � Nc � Nb � Na � Tc � Tb � Ta �

�����������������������������������������

If D1 is 1, R14 acts as output register (RS232 output: bit 2 is CTS,

bit 3 is data output); when it is 0 R14 acts as input register (bit 6

is DTR [3]).  D2 is ignored as the AY chip has only one I/O register;

the bit and its corresponding register are present however.  Reading

R15 in input mode always yields 255.  Changes made to R7 take effect

immediately.

The noise and tone output of a channel is combined in the mixer in the

following way:

Output_A = (Tone_A OR Ta) AND (Noise OR Na)

Here Tone_A is the binary output of tone generator A, and Noise is the

binary output of the noise generator.  Note that setting both Ta and Na

to 1 produces a constant 1 as output.  Also note that setting both Ta

and Na to 0 produces bursts of noise and half-periods of constant

output 0.

Each binary tone channel output is fed to a separate volume generator.

Each volume generator is controlled by its amplitude register (R8 for

channel A) and the 4-bit output of the envelope controller.

Bit      7    6    5    4    3    2    1    0

���������������������������������������Ŀ

R8    �    �    �    � Ev � V3 � V2 � V1 � V0 �

�����������������������������������������

If Ev=0, the current volume is given by V3V2V1V0.  If Ev=1, the current

volume is given by the output of the envelope generator.  The volume

controller produces an output voltage proportional to its channel's

binary output value times the current volume.  These analogue outputs

are then added together to give the final output (this is done outside

the chip actually).  Note that even when a channel is disabled (say Ta

= Na = 1), changing the volume level changes the final output, as the

binary tone channel output is constant and equal to 1, not 0.  Changes

to the amplitude registers take effect immediately.

The envelope generator is controlled by R11, R12 and R13.

Bit     7  6  5  4      3         2         1        0

��������������������������������������������������Ŀ

R13   �  �  �  �  � Continue � Attack � Alternate � Hold �

����������������������������������������������������

The envelope generator contains a 16-bit up counter, operated at a

frequency of 110830 Hz.  The lowest frequency attainable by the

envelope clock is therefore 1.7 Hz.  If R12R11 contains 0, the clock

runs at 110830 Hz; otherwise it runs at 110830/R12R11 Hz.  The envelope

generator is reset by writing to R13 (but not by writing to R11 or

R12); otherwise it works just as the other counters.

Each time the envelope generator up counter is reset, it produces a

envelope clock tick.  A 'period' is the time taken by 16 clock ticks.

The output of the envelope generator during the first period is as

follows.  If Attack = 1, the output starts off at 0 and at each clock

tick is increased by 1 until it reaches 15.  If Attack = 0, the output

starts off at 15 and is falls to 0.

The output in the subsequent periods is 0 if Continue = 0.  Otherwise,

first the (internal) 'Attack' bit is toggled if Alternate is set,

otherwise it doesn't change.  How if Hold = 1, the output is a constant

15 in this and subsequent periods if Attack = 1, otherwise it is a

constant 0.  If Hold = 0, the envelope generator behaves just as in the

first period (except that Attack has possibly changed).  To sum it up:

0,1,2,3  \__________     single decay then off

4,5,6,7  /|_________     single attack then off

8        \|\|\|\|\|\     repeated decay

9        \__________     single decay then off

10       \/\/\/\/\/\     repeated decay-attack

_________

11       \|              single decay then hold

12       /|/|/|/|/|/     repeated attack

__________

13       /               single attack then hold

14       /\/\/\/\/\/     repeated attack-decay

15       /|_________     single attack then off

If the envelope generator is used to generate a tone, its frequency is

either 110830/(16*R12R11) Hz (R13 = 8 or 12) or 110830/(32*R12R11) Hz

(R13 = 10 or 14).

[1] This has been checked by Pierre Guerrier using an oscilloscope.

[2] The algorithm used for the pseudo random output is not known.  It

seems not to be too good, as sounds vaguely similar to tape loading

sounds are audible if you set R6 to 31.

[3] I don't know which bit is data input; it is probably not bit 3.

5.4  The ZX Printer

As always, very Sinclair, this is a simple but ingenious piece of

equipment.  The ZX Printer is controlled through one I/O port, namely

#FB.  It is decoded using A2 only.  Port #FB:

Bit      7    6    5    4    3    2    1    0

���������������������������������������Ŀ

READ  �styl�  0 �    �    �    �    �    � enc�

�����������������������������������������

���������������������������������������Ŀ

WRITE �Data�    �    �    �    � Mot�Slow�    �

�����������������������������������������

To print, first read port #FB and check that bit 6 is 0 to make sure

that the printer is attached.  The printer motor is started by writing

a byte 0 to #FB (Mot=0).  Then check bit 7 until it goes high; this

means that the printer head is in the starting position.  A line is

printed as follows.  Wait for bit 0 (encoder) to go high, and write a

bit of data (1=black dot), and do this 256 times.  The Spectrum ROM

makes the 'Slow' bit 1 in the last two lines; this ensures that the

printer head stops at the start of the new line, instead of halfway

into it.  Stop the printer by writing a byte 4.  See #0EF4 in the ROM

for the relevant routine.

The emulator returns #FF on a port #FB IN if there is no printer

attached to the LPT port selected for ZX Printer output.  If the

printer is off-line, busy, or out of paper, the emulator will usually

return #FF too, so that the Spectrum program will not even consider to

print, unless you set option -xj, as then the emulator will return 1 so

that the program will try to print, and will wait for the printer to go

on line.  The reason for including the -xj switch is that usually the

status lines of the printer are not too reliable, so that a Spectrum

program may halt on a COPY statement even if there's no printer

attached (e.g. with SuperSpy, which makes screendumps on the ZX Printer

without asking).  In the inner printing loop, the ROM does not check

for BREAKs, so that the encoder bit is always high to prevent locking

the emulated Spectrum.  If the printer is happy to accept data, the

emulator returns #81.

5.5  The Interface I

The Interface I is quite complicated.  It uses three different I/O

ports, and contains logic to page and unpage an 8K ROM if new commands

are used.  I won't be very detailed here; you could refer to the source

code of the emulator if you want to know some details, or read the

'Spectrum Shadow ROM Disassembly' by Gianlura Carri, published by

Melbourne House - but don't expect the same level of detail as of Ian

Logan and Frank O'Hara in their Rom disassembly book.

The ROM is paged if the processor executes the instruction at ROM

address 0008 or 1708 hexadecimal, the error and close# routines.  It is

inactivated when the Z80 executes the RET at address 0700.

I/O Port E7 is used to send or receive data to and from the microdrive.

Accessing this port will halt the Z80 until the Interface I has

collected 8 bits from the microdrive head; therefore, it the microdrive

motor isn't running, or there is no formatted cartridge in the

microdrive, the Spectrum hangs.  This is the famous 'IN 0 crash'.

Port EF is used for several things:

Bit    7   6    5    4    3    2    1     0

���������������������������������������Ŀ

READ�   �   �    �busy� dtr �gap� sync�write�

�   �   �    �    �     �   �     �prot.�

���������������������������������������Ĵ

WRITE�   �   �wait� cts�erase�r/w�comms�comms�

�   �   �    �    �     �   � clk � data�

�����������������������������������������

Bits DTR and CTS are used by the RS232 interface.  The WAIT bit is used

by the Network to synchronise, GAP, SYNC, WR_PROT, ERASE, R/_W, COMMS

CLK and COMMS DATA are used by the microdrive system.  If the

microdrive is not being used, the COMMS DATA output selects the

function of bit 0 of out-port F7:

Bit      7    6   5   4   3   2   1       0

������������������������������������������Ŀ

READ�txdata�   �   �   �   �   �   �    net    �

�      �   �   �   �   �   �   �   input   �

������������������������������������������Ĵ

WRITE�      �   �   �   �   �   �   �net output/�

�      �   �   �   �   �   �   �   rxdata  �

��������������������������������������������

TXDATA and RXDATA are the input and output of the RS232 port.  COMMS

DATA determines whether bit 0 of F7 is output for the RS232 or the

network.

5.6  The SamRam

The SamRam contains a 32K static CMOS Ram chip, and some I/O logic for

port 31.  If this port is read, it returns the position of the

joystick, as a normal Kempston joystickinterface would.  If written to,

the port controls a programmable latch chip (the 74LS259) which

contains 8 latches:

Bit    7   6   5   4   3   2   1   0

�������������������������������Ŀ

WRITE�   �   �   �   �  address  �bit�

���������������������������������

The address selects on of the eight latches; bit 0 is the new state of

the latch.  The 16 different possibilities are collected in the diagram

below:

OUT 31,   �  Latch  � Result

�������������������������������������������������������������

0     �    0    � Switch on write protect of CMOS RAM

1     �    "    � Writes to CMOS RAM allowed

2     �    1    � turn on CMOS RAM (see also 6/7)

3     �    "    � turn off CMOS RAM (standard Spec.  ROM)

4     �    2    � -

5     �    "    � Ignore all OUT's to 31 hereafter

6     �    3    � Select CMOS bank 0 (Basic ROM)

7     �    "    � Select CMOS bank 1 (Monitor,...)

8     �    4    � Select interface 1

9     �    "    � Turn off IF 1 (IF1 rom won't be paged)

10     �    5    � Select 32K ram bank 0 (32768-65535)

11     �    "    � Select 32K ram bank 1 (32768-65535)

12     �    6    � Turn off beeper

13     �    "    � Turn on beeper

14     �    7    � -

15     �    "    � -

At reset, all latches are 0.  If an OUT 31,5 is issued, only a reset

will give you control over the latches again.  The write protect latch

is not emulated; you're never able to write the emulated CMOS ram in

the emulator.  Latch 4 pulls up the M1 output at the expansion port of

the Spectrum.  The Interface I won't page its ROM anymore then.

5.7  The Multiface 128

This device consists of an 8K rom, paged into the area 0-8191, and 8K

of ram, paged into 8192-16383, and a latch storing the screen select

bit of the Spectrum 128K (bit 3 of OUT-port #7FFD), which would

otherwise be impossible to know (the Disciple requires the user to

press Y or N according to whether the screen changed at some point of

the snapshot procedure, in order to get hold of this bit.)

The Multiface rom and ram pages have highest priority, higher than both

the ordinary rom (48K or 128K) or the Interface 1, Disciple or Plus D

roms.

The Multiface memory is paged in when an NMI occurs, or when the

processor reads from IN address #BF.  Bit 7 of the returned byte is the

screen select bit (or, on 48K machines, the value of bit 3 of the byte

last written to port #7FFD - unlikely to be useful.)  Other bits follow

the floating bus.

The Multiface memory is paged out by reading from port #3F.  The

Multiface does not put bits on the bus.

5.8  The Z80 microprocessor

The Z80 processor is quite straightforward, and contains to my

knowledge no interesting bugs or quirks.  However, it has some

undocumented features.  Some of these are quite useful, and some are

not, but since many programs use the useful ones, and a few programs

use the weird ones, I tried to figure them out and emulate them as best

as I could.  There is a Z80 emulator around, intended as a CP/M

emulator, which halts the program if an undocumented opcode is

encountered.  I don't think this makes sense.  ZiLOG doesn't dictate

the law, the programs which use the processor's features do!

In section 5.1 there is some information on Z80 interrupt timings in

different modes.

Most Z80 opcodes are one byte long, not counting a possible byte or

word operand.  The four opcodes CB, DD, ED and FD are 'shift' opcodes:

they change the meaning of the opcode following them.

There are 248 different CB opcodes.  The block CB 30 to CB 37 is

missing from the official list.  These instructions, usually denoted by

the mnemonic SLL, Shift Left Logical, shift left the operand and make

bit 0 always one.  Bounder and Enduro Racer use them, to name just two.

The SamRam monitor can disassemble these and uses the mnemonic SLL.

These instructions are quite commonly used.

The DD and FD opcodes precede instructions using the IX and IY

registers.  If you look at the instructions carefully, you see how they

work:

2A nn           LD HL,(nn)

DD 2A nn        LD IX,(nn)

7E              LD A,(HL)

DD 7E d         LD A,(IX+d)

A DD opcode simply changes the meaning of HL in the next instruction.

If a memory byte is addressed indirectly via HL, as in the second

example, a displacement byte is added.  Otherwise the instruction

simply acts on IX instead of HL.  (A notational awkwardness, that will

only bother assembler and disassembler writers: JP (HL) is not

indirect; it should have been denoted by JP HL.)  If a DD opcode

precedes an instruction that doesn't use the HL register pair at all,

the instruction is executed as usual.  However, if the instruction uses

the H or L register, it will now use the high or low halves of the IX

register!  Example:

44              LD B,H

FD 44           LD B,IYh

These types of inofficial instructions are used in many programs.  By

the way, many DD or FD opcodes after each other will effectively be

NOPs, doing nothing except repeatedly setting the flag 'treat HL as IX'

(or IY) and taking up 4 T states.  Note that the FD or DD 'opcode' is

treated as part of an instruction, so that the Z80 cannot be interrupted

during execution of such a block.  (This last remark was not checked;

it was remarked by someone on comp.sys.sinclair)

When (parts of) HL are used both as source and target, above rule is

ambiguous.  The special cases are: LD H,(IX+d) and LD L,(IX+d), and

LD IXl,IXl / LD IXl,IXh / LD IXh,IXl / LD IXh,IXh.  Things like

LD IXh,(IX+d) or LD H,IXh do not exist.

Two instructions do not obey the DD/FD rule.  They are EX DE,HL and

EXX.  These instructions take 4 T states to execute, and are therefore

most probably implemented by toggling a flag that changes the reference

to the registers; no data is actually being moved, which explains why

they cannot operate on IX or IY.

The doubly-shifted opcodes that start with DD CB and DD ED also do not

follow the standard rule.  If a DD or FD precedes an ED instruction,

it is ignored.  ED instructions never operate on the IX or IY register.

With CB instructions, the situation is more interesting.  Every DD CB

instruction operates on (IX+nn), but also copies the result to the

specified register, except when it is (HL).  For example,

CB CE           SET 0,(HL)

CB C0           SET 0,B

DD CB nn CE     SET 0,(IX+nn)

DD CB nn C0     SET 0,(IX+nn) ; copy result to B

(The information about the inofficial CB instructions was given to me

by Arnt Gulbrandsen, and originated from David Librik.)

There are a number of inofficial ED instructions, but none of them are

very useful.  The ED opcodes in the range 00-3F and 80-FF (except for

the block instructions of course) do nothing at all but taking up 8 T

states and incrementing the R register by 2.  Most of the unlisted

opcodes in the range 40-7F do have an effect, however.  The complete

list: (* = not official)

ED40   IN B,(C)                 ED60   IN H,(C)

ED41   OUT (C),B                ED61   OUT (C),H

ED42   SBC HL,BC                ED62   SBC HL,HL

ED43   LD (nn),BC               ED63 # LD (nn),HL

ED44   NEG                      ED64 * NEG

ED45   RETN                     ED65 * RET

ED46   IM 0                     ED66 * IM 0

ED47   LD I,A                   ED67   RRD

ED48   IN C,(C)                 ED68   IN L,(C)

ED49   OUT (C),C                ED69   OUT (C),L

ED4A   ADC HL,BC                ED6A   ADC HL,HL

ED4B   LD BC,(nn)               ED6B # LD HL,(nn)

ED4C * NEG                      ED6C * NEG

ED4D   RETI                     ED6D * RET

ED4E * IM 0                     ED6E * IM 0

ED4F   LD R,A                   ED6F   RLD

ED50   IN D,(C)                 ED70 # IN (C)

ED51   OUT (C),D                ED71 * OUT (C),0

ED52   SBC HL,DE                ED72   SBC HL,SP

ED53   LD (nn),DE               ED73   LD (nn),SP

ED54 * NEG                      ED74 * NEG

ED55 * RET                      ED75 * RET

ED56   IM 1                     ED76 * IM 1

ED57   LD A,I                   ED77 * NOP

ED58   IN E,(C)                 ED78   IN A,(C)

ED59   OUT (C),E                ED79   OUT (C),A

ED5A   ADC HL,DE                ED7A   ADC HL,SP

ED5B   LD DE,(nn)               ED7B   LD SP,(nn)

ED5C * NEG                      ED7C * NEG

ED5D * RET                      ED7D * RET

ED5E   IM 2                     ED7E * IM 2

ED5F   LD A,R                   ED7F * NOP

Guenter Woigk remarked that the three instructions marked # (which were

previously marked *) are documented by ZiLog, though not normally used.

The ED63 and ED6B instructions have shorter and faster equivalents.

The ED70 instruction reads from port (C), just like the other

instructions, but throws away the result.  It does change the flags in

the same way as the other IN instructions, however.  The ED71

instruction OUTs a byte zero to port (C), interestingly.  These

instructions 'should', by regularity of the instruction set, use (HL)

as operand, but since from the processor's point of view accessing

memory or accessing I/O devices is the same thing except for activation

of the /IORQ line instead of the /MREQ line, and since the Z80 does not

access memory twice in one instruction (disregarding instruction fetch

of course) it can't fetch or store the data byte.  (A hint in this

direction is that, even though the NOP-synonyms LD B,B, LD C,C etcetera

do exist, LD (HL),(HL) is absent and replaced by the HALT instruction.)

The instructions ED 4E and ED 6E are IM 0 equivalents: when FF was put

on the bus (physically) at interrupt time, the Spectrum continued to

execute normally, whereas when an EF (RST #28) was put on the bus it

crashed, just as it does in that case when the Z80 is in the official

interrupt mode 0.  In IM 1 the Z80 just executes a RST #38 (opcode FF)

no matter what is on the bus.

The RETI instruction is functionally exactly equivalent to the RET

instruction.  It is used only to signify the end of an interrupt

routine to an external hardware device (read: the Z80 PIO).  The RETN

however is different from RET in that it resets IFF1 to the current

value of IFF2.  Now IFF1 and IFF2 are usually equal (and become equal

after DI and EI and after a maskable interrupt has been accepted).

They're different only if an NMI occurs when interrupts are enabled;

then IFF1 is off, and IFF2, holding the previous state of the interrupt

flip flop, is on, signifying that interrupts were enabled before the

non-maskable interrupt.  Since the state of IFF2 can be read by using

LD A,R and LD A,I, the RETN instruction is not used much in Spectrum

ROM software, and it is utterly useless in normal software.  In other

words, I have not tried to figure out whether the unofficial RET's are

RETI's or RETN's.  One can make an educated guess though.

About the R register.  This is not really an undocumented feature,

although precise explanations were hard to find.  The R register is a

counter that is updated every instruction, where DD, FD, ED and CB are

to be regarded as separate instructions.  So shifted instructions will

increase R by two.  There's an exception: doubly-shifted opcodes, the

DDCB and FDCB ones, increase R by two too.  LDI increases R by two,

LDIR increases it by 2 times BC, as does LDDR etcetera.  The sequence

LD R,A / LD A,R increases A by two. The highest bit of the R register

is never changed (except possibly by LD R,A of course).  This is

because in the old days everyone used 16 Kbit chips.  Inside the chip

the bits were arranged in a 128x128 matrix, and needed a 7 bit refresh

cycle.  Probably for this reason ZiLOG decided to count only the lowest

7 bits.  If the R register emulation is switched on the R register will

behave as is does on a real Spectrum; if it is off it will (except for

the upper bit) act as a random generator.

You can easily check that the R register is really crucial to memory

refresh.  Assemble this program:

ORG 32768

DI

LD B,0

L1  XOR A

LD R,A

DEC HL

LD A,H

OR L

JR NZ,L1

DJNZ L1

EI

RET

It will take about three minutes to run.  Look at the upper 32K of

memory, for instance the UDG graphics.  It will have faded.  Only the

first few bytes of each 256 byte block will still contain zeros,

because they were refreshed during the execution of the loop.  The ULA

took care of the refreshing of the lower 16K.  (This example won't work

on the emulator of course!)

Then there's one other dark corner of the Z80 which has its effect on

programs like Sabre Wulf, Ghosts'n Goblins and Speedlock.  The Mystery

of the Undocumented Flags!

Bit 3 and 5 of the F register are not used.  They can contain

information, as you can readily figure out by using PUSH AF and POP AF.

Furthermore, sometimes their values change.  I found the following

empirical rule:

The values of bit 7, 5 and 3 follow the values of the

corresponding bits of the last 8 bit result of an instruction

that changed the usual flags.

For instance, after an ADD A,B those bits will be identical to the bits

of the A register.  (Bit 7 of F is the sign flag, and fits the rule

exactly).  An exception is the CP x instruction (x=register, (HL) or

direct argument).  In that case the bits are copied from the argument.

If the instruction is one that operates on a 16 bit word, the 8 bits of

the rule are the highest 8 bits of the 16 bit result - that was to be

expected since the S flag is extracted from bit 15.

Ghosts'n Goblins use the undocumented flag due to a programming error.

The rhino in Sabre Wulf walks backward or keeps running in little

circles in a corner, if the (in this case undocumented) behaviour of

the sign flag in the BIT instruction isn't right.  I quote:

AD86    DD CB 06 7E        BIT 7,(IX+6)

AD8A    F2 8F AD           JP P,#AD8F

An amazing piece of code!  Speedlock does so many weird things that

everything must be exactly right for it to run.  Finally, the '128 rom

uses the AF register to hold the return address of a subroutine for a

while.  To keep all programs happy, and still have a fast emulator, I

had to make a compromise.  The undocumented flags are not always

emulated right, but they are most of the time.  Not telling you when

not.

Now for the emulated Z80.  I have added eight instructions, to speed up

the RS232 input and output of the Interface I and several things of the

SamRam.  These opcodes, ED F8 to ED FE are of little use to any other

program.  ED FF is a nice one: it returns you to DOS immediately.  I

used it for debugging purposes, and it is also used in TAP2TAPE and

SAMLIST.

The opcode ED FB, which is used by the SamRam, is now also used to use

multi-load games on the emulator.  If the emulator encounters the

opcode ED FB in RAM (above 16384), it loads a block of code into memory

at address HL.  The data is first looked for in the .Z80 or .SLT

snapshot file last loaded (see description of the .Z80 file format); if

it is not found there, the emulator looks for a .DAT file with the same

name as the snapshot last loaded, and the decimal value of the A

register appended to it (dropping letters from the snapshot name if

required to make it at most 8 characters long).  If this file is not

found either, a window is popped up to inform the user of the value of

the A register, and allow him to supply a .DAT file (or .Z80/.SLT file)

himself.

When the emulator fails to load a block, it complements the carry flag,

so that the running Spectrum program can decide what to do.  Note that

versions of Z80 prior to v3.04 did not change the flags, so that on old

versions (and on other emulators) loading will always seem to succeed.

5.9  File formats

This section describes the format of the files used by the emulator.

ROMS.BIN:

---------

00000-03fff     Ordinary Spectrum rom

04000-05fff     Interface I rom (8K)

06000-09fff     First SamRam rom (contains BASIC)

0a000-0dfff     Second SamRam rom (contains monitor,...)

0e000-11fff     First Spectrum 128K rom (active at RESET)

12000-15fff     Second Spectrum 128K rom (contains BASIC)

16000-19fff     Disciple rom, system file 3b, Epson printer code

1a000-1dfff     Disciple rom, system file 3b, HP printer code

1e000-1ffff     Multiface rom (8K)

The ordinary rom has not been modified.  The Interface I rom has

undergone some modifications, to speed up the RS232 input/output

routines.  If you don't like this, or want to use another version of

the Interface I, you could put that code at the right place in the

ROMS.BIN file.  The interface I should work properly, although the

RS232 will be slower (always FORMAT the "b" or "t" channel at 19200

baud, by the way, if you replace the rom code, there's no point in

waiting for nothing.)  The microdrive routines have not been modified

in any way.  Here are the changes of the Interface I rom:

Address: Old: New:           Address: Old: New:

0B9E     ED   ED             0D20     FB   00

0B9F     5B   FC             0D2A     37   ED

0BA0     C3   F5             0D2B     F3   FD

0BA1     5C   C3             0D2C     CE   18

0BA2     21   34             0D2D     00   10

0BA3     20   0C             0D4C     FB   00

These changes are not likely to cause problems; there are several

versions of the Interface I rom around, and program developers know

this.  It is also a bit pointless to check whether the Interface I rom

hasn't been modified; who would put his snapshot software in there

anyway, and that's what those people are afraid of.

The first and second SamRam rom have been modified more extensively.

The biggest problem was that switching the upper 32K ram bank is very

fast in reality, but on the PC two blocks of 32K bytes had to be REP

MOVSWded (or the EMS emulator be called).  But since no programs know

of the SamRam code anyway, this won't cause any more problems it

wouldn't already cause either.

The two Spectrum 128 roms have not been modified, and neither have the

Disciple roms or the Multiface rom.  The Disciple roms, as they appear

in the ROMS.BIN file, do have system files pre-loaded however.

.TAP FILES:

-----------

The .TAP files contain blocks of tape-saved data.  All blocks start

with two bytes specifying how many bytes will follow (not counting the

two length bytes).  Then raw tape data follows, including the flag and

checksum bytes.  The checksum is the bitwise XOR of all bytes including

the flag byte.  For example, when you execute the line SAVE "ROM" CODE

0,2 this will result:

|------ Spectrum-generated data -------|       |---------|

13 00 00 03 52 4f 4d 7x20 02 00 00 00 00 80 f1 04 00 ff f3 af a3

^^^^^...... first block is 19 bytes (17 bytes+flag+checksum)

^^... flag byte (A reg, 00 for headers, ff for data blocks)

^^ first byte of header, indicating a code block

file name ..^^^^^^^^^^^^^

header info ..............^^^^^^^^^^^^^^^^^

checksum of header .........................^^

length of second block ........................^^^^^

flag byte ............................................^^

first two bytes of rom .................................^^^^^

checksum (checkbittoggle would be a better name!).............^^

The emulator will always start reading bytes at the beginning of a

block.  If less bytes are loaded than are available, the other bytes

are skipped, and the last byte loaded is used as checksum.  If more

bytes are asked for than exist in the block, the loading routine will

terminate with the usual tape-loading-error flags set, leaving the

error handling to the calling Z80 program.

Note that it is possible to join .TAP files by simply stringing them

together, for example COPY /B FILE1.TAP + FILE2.TAP ALL.TAP

For completeness, I'll include the structure of a tape header.  A

header always consists of 17 bytes:

Byte    Length  Description

0       1       Type (0,1,2 or 3)

1       10      Filename (padded with blanks)

11      2       Length of data block

13      2       Parameter 1

15      2       Parameter 2

The type is 0,1,2 or 3 for a Program, Number array, Character array or

Code file.  A screen$ file is regarded as a Code file with start

address 16384 and length 6912 decimal.  If the file is a Program file,

parameter 1 holds the autostart line number (or a number >=32768 if no

LINE parameter was given) and parameter 2 holds the start of the

variable area relative to the start of the program.  If it's a Code

file, parameter 1 holds the start of the code block when saved, and

parameter 2 holds 32768.  For data files finally, the byte at position

14 decimal holds the variable name.

.MDR FILES:

-----------

The emulator uses a cartridge file format identical to the 'Microdrive

File' format of Carlo Delhez' Spectrum emulator Spectator for the QL,

who devised the format.  This format is now also supported by XZX of

Des Harriot.  The following information is adapted from Carlo's

documentation.  It can also be found in the 'Spectrum Microdrive Book',

by Ian Logan (co-writer of the excellent 'Complete Spectrum ROM

Disassembly').

A cartridge file contains 254 'sectors' of 543 bytes each, and a final

byte flag which is non-zero is the cartridge is write protected, so the

total length is 137923 bytes.  On the cartridge tape, after a GAP of

some time the Interface I writes 10 zeros and 2 FF bytes (the

preamble), and then a fifteen byte header-block-with-checksum.  After

another GAP, it writes a preamble again, with a 15-byte record-

descriptor-with-checksum (which has a structure very much like the

header block), immediately followed by the data block of 512 bytes, and

a final checksum of those 512 bytes.  The preamble is used by the

Interface I hardware to synchronise, and is not explicitly used by the

software.  The preamble is not saved to the microdrive file:

offset length name    contents

0      1   HDFLAG   Value 1, to indicate header block

1      1   HDNUMB   sector number (values 254 down to 1)

2      2            not used

4     10   HDNAME   microdrive cartridge name (blank padded)

14      1   HDCHK    header checksum (of first 14 bytes)

15      1   RECFLG   - bit 0: always 0 to indicate record block

- bit 1: set for the EOF block

- bit 2: reset for a PRINT file

- bits 3-7: not used (value 0)

16      1   RECNUM   data block sequence number (value starts at 0)

17      2   RECLEN   data block length (<=512, LSB first)

19     10   RECNAM   filename (blank padded)

29      1   DESCHK   record descriptor checksum (of previous 14 bytes)

30    512            data block

542      1   DCHK     data block checksum (of all 512 bytes of data

block, even when not all bytes are used)

---------

254 times

(Actually, this information is 'transparent' to the emulator.  All it

does is store 2 times 254 blocks in the .MDR file as it is OUTed,

alternatingly of length 15 and 528 bytes.  The emulator does check

checksums, see below; the other fields are dealt with by the emulated

Interface I software.)

A used record block is either an EOF block (bit 1 of RECFLG is 1) or

contains 512 bytes of data (RECLEN=512, i.e.  bit 1 of MSB is 1).  An

empty record block has a zero in bit 1 of RECFLG and also RECLEN=0.  An

unusable block (as determined by the FORMAT command) is an EOF block

with RECLEN=0.

The three checksums are calculated by adding all the bytes together

modulo 255; this will never produce a checksum of 255.  Possibly, this

is the value that is read by the Interface I if there's no or bad data

on the tape.

In normal operation, all first-fifteen-byte blocks of each header or

record block will have the right checksum.  If the checksum is not

right, the block will be treated as a GAP.  For instance, if you type

OUT 239,0 on a normal Spectrum with interface I, the microdrive motor

starts running and the cartridge will be erased completely in 7

seconds. CAT 1 will respond with 'microdrive not ready'.  Try it on the

emulator...

.OUT FILES:

-----------

These files are produced when logging OUTs; see menu option O in the

Extra Functions menu.  For the specified I/O ports, all OUTs to these

ports are recorded in the .OUT file, together with the exact time at

which the OUTs were executed.

An .OUT file consists of a string of 5-byte blocks.  The first word is

the timing word; it has a value between 0 and 17471 inclusive (or

between 0 and 17726 inclusive when a 128K Spectrum is emulated), and a

unit value corresponds to 1 T state (=1/3494400 s).  After this, the

OUT port that was written to follows, then the value OUTed itself:

Offset  Length  Description

0       2       Time (0-17471 or 0-17726)

2       2       Port address

4       1       Value

Every 1/200th of an emulated second, that is, every 69888/4=17472 T

states (or 70908/4=17727 T states on 128K Spectrums), a time-wraparound

block is written to the .OUT file:

Offset  Length  Description

0       2       Flag word #FFFF indicating wraparound-block

2       2       Length of preceding block (17472 or 17727 T)

4       1       Not used

So even when the Spectrum does not OUT to the logged ports at all, 1000

bytes get written to the log file every second.

By default, an OUT to an even I/O address which does not change the

state of the MIC and EAR outputs is not written to the .OUT file, to

save disk space when recording music.  If you want all OUTs, specify

-xg on the command line.

The .OUT files are also used to make a simple trace of a running

Spectrum program.  Specify -xy on the command line; as soon as you

activate OUT logging, a trace is dumped also.  For each instruction

encountered during emulation, the following block is written to the

.OUT file:

Offset  Length  Description

0       2       Flag word (#FFFE)

2       2       Program counter

5       1       A register

Furthermore, no time-wraparound blocks are written to the .OUT file

when tracing.  HALT instructions (118 decimal) are special in that they

do not generate a block in the log file; this is to make comparisons

between different logs of the same program easier.  Admittedly, this is

a very crude way of tracing a program, but it's better than nothing,

and very useful very occasionally.

Map files

---------

Map files are those that are produced via option -0m; see section 2.19

in Z80.DOC.  These files have no default extension; .MAP seems a natural

suggestion.  These files are meant to be used with a future version of

Leslie Styles' DSNA (a snapshot disassembler; you can find it on

ftp.nvg.unit.no).

Map files are 8192 bytes long.  Each byte represents 8 addresses in

Spectrum memory, from #0000 to #FFFF.  Note that the rom is also

mapped.  A one at bit-position k in byte n means that at least once

during the time the emulator emulated, it emulated the opcode it found

at address 8n+k.  The shifted opcodes will only have the shift code

marked (though the Z80 chip is, I think, in machine cycle 1, M1, during

both the shift fetch and the secondary opcode fetch).  Due to the way

inofficial DD and FD instructions are implemented, a sequence of DD's

is considered as one very long instruction, and will result in only the

address of the first DD that was executed being tagged.

You should not use Hi-res color emulation during opcode mapping; they

use the same buffer area.  HRC emulation still works, but destroys the

mapping buffer.  Also, option -xu (disable HCR emulation to use less

memory) will disable opcode mapping.  Finally, the mapping procedure

does not take memory paging into account, and it is therefore not very

practical in 128K modes, in SamRam mode, and for mapping the execution

of the shadow roms (128K, SamRam, Interface I, Multiface).  It works

best for mapping the ordinary rom and ordinary 48K Spectrum programs.

Also note that the map is never cleared, not even when another snapshot

is loaded.

The best way to use this feature is the following.  Load the program

you want to disassemble, and let it run to the main starting point, so

that all decoding, decompressing, and moving around has probably been

done.  Then save the snapshot ('program.z80'), and start the emulator

with 'z80 -0m program.map program.z80'.  Run it for a while, but be

sure never to reset the Spectrum, or to enter the SamRam or Multiface,

or load another snapshot.  After a while, quit the emulator

(Ctrl-Break), and you'll find the file 'program.map' contain the

locations of the opcodes executed.  By tracing the code blocks found,

and also taking the conditional branches that were left alone during

execution, most of the code can probably be found.

This option has been added after reading a remark of Leslie Styles

(lsm@soton.ac.uk), who had been trying to make his disassembler figure

out which parts of memory were actual code, and which were data.  It

seemed to me that the only reliable way was to actually emulate the

code.  For almost all programs this will be sufficient.  It however

still does not take into account the possibility of self-modifying

code.  For completely general programs, the approach that Arnt

Gulbrandsen followed in his JPP (also keeping a map of executed opcode

locations, but invalidating these if necessary when data is written to

memory)  is the only right solution.  The current approach however was

so simple to implement that I thought it to be worthwhile.

.SCR FILES:

-----------

.SCR files are memory dumps of the first 6912 bytes of the Spectrum

memory.  A coordinate (x,y), x between 0 and 255 and y between 0 and

192, (0,0) being the upper left corner of the screen, corresponds to

the pixel address

16384+INT (x/8)+1792*INT (y/64)-2016*INT (y/8)+256*y

I admit this is not quite the clearest way to explain the organization

of Spectrum's video memory, but with a bit or (hard) thinking you can

extract from above formula all information you need...  The lowest

three bits of x determine which bit of this address corresponds to the

pixel (x,y).  This bit-map constitutes the larger part of the screen

memory, 256*192/8=6144 bytes.  The final 768 bytes are attribute bytes.

The address of the attribute byte corresponding to pixel (x,y) is

22528+INT (x/8)+32*INT (y/8)

The lowest three bits of the attribute byte control the foreground

colour (the colour of the pixel if the corresponding bit in the bitmap is

set), bits 3-5 control the background colour, bit 6 is the bright bit

and bit 7 is the flash bit - if it is set, every 16/50th of a second

the ULA effectively flips the foreground and background colours.

.DAT files:

-----------

These files are used to store level data; blocks of memory that are

loaded when the opcode ED FB is executed.  The block is loaded at the

address pointed to by the HL register.  The level number is in the A

register.  If the emulator fails to load a level data block, it

complements the carry flag.  Versions of the emulator prior to v3.04

did not do this.

When an ED FB opcode is encountered, the emulator first looks for the

snapshot file last loaded, and checks whether it contains the level

requested (see the description of the .Z80 format below).  If this

fails, it looks for a .DAT file of the level; the name of this file is

derived from the name of the last snapshot loaded by appending the

(decimal) value of the A register to the name of the snapshot file,

dropping characters from the original snapshot name to make the total

length 8 characters if necessary.  .DAT files simply contain the plain

level data (in contrast to the level data in .Z80 files, which are

compressed).

.Z80 and .SLT files:

--------------------

The old .Z80 snapshot format (for version 1.45 and below) looks like

this:

Offset  Length  Description

0       1       A register

1       1       F register

2       2       BC register pair (LSB, i.e. C, first)

4       2       HL register pair

6       2       Program counter

8       2       Stack pointer

10      1       Interrupt register

11      1       Refresh register (Bit 7 is not significant!)

12      1       Bit 0  : Bit 7 of the R-register

Bit 1-3: Border colour

Bit 4  : 1=Basic SamRom switched in

Bit 5  : 1=Block of data is compressed

Bit 6-7: No meaning

13      2       DE register pair

15      2       BC' register pair

17      2       DE' register pair

19      2       HL' register pair

21      1       A' register

22      1       F' register

23      2       IY register (Again LSB first)

25      2       IX register

27      1       Interrupt flipflop, 0=DI, otherwise EI

28      1       IFF2 (not particularly important...)

29      1       Bit 0-1: Interrupt mode (0, 1 or 2)

Bit 2  : 1=Issue 2 emulation

Bit 3  : 1=Double interrupt frequency

Bit 4-5: 1=High video synchronisation

3=Low video synchronisation

0,2=Normal

Bit 6-7: 0=Cursor/Protek/AGF joystick

1=Kempston joystick

2=Sinclair 2 Left joystick (or user

defined, for version 3 .Z80 files)

3=Sinclair 2 Right joystick

Because of compatibility, if byte 12 is 255, it has to be regarded as

being 1.  Following this header block of 30 bytes the 48K bytes of

Spectrum memory are stored, in a compressed format (if bit 5 of byte 12

is set).

The compression method is this.  Repetitions of at least five equal

bytes are replaced by the four-byte code ED ED xx yy, which stands for

"byte yy repeated xx times".  Only sequences of length at least 5 are

coded. The exception is sequences consisting of ED's; if they are

encountered, even two ED's are encoded into ED ED 02 ED.  Finally,

every byte directly following a single ED is not taken into a block,

for example ED 6*00 is not encoded into ED ED ED 06 00 but into ED 00

ED ED 05 00.  The block is terminated by an end marker, 00 ED ED 00.

That's the format of .Z80 files as used by versions up to 1.45.

Starting from version 2.0, a different format is used, since from then

on also 128K snapshots had to be supported.  This new format is used

for all snapshots (48K or 128K).

Version 2.01 and 3.0x .Z80 files start with the same 30 byte header

that old .Z80 files used.  Bit 4 and 5 of the flag byte (offset 12)

have no meaning anymore, and the program counter (offset 6 and 7) are

zero to signal a version 2.01 (or later) snapshot file.

After the first 30 bytes, the additional header follows:

Offset  Length  Description

* 30      2       Length of additional header block (see below)

* 32      2       Program counter

* 34      1       Hardware mode (see below)

* 35      1       If in SamRam mode, bitwise state of 74ls259.

For example, bit 6=1 after an OUT 31,13 (=2*6+1)

If in 128 mode, contains last OUT to 7ffd

* 36      1       Contains 0FF if Interface I rom paged

* 37      1       Bit 0: 1 if R register emulation on

Bit 1: 1 if LDIR emulation on

* 38      1       Last OUT to fffd (soundchip register number)

* 39      16      Contents of the sound chip registers

55      2       Low T state counter

57      1       Hi T state counter

58      1       Flag byte used by Spectator (QL spec. emulator)

Ignored by Z80 when loading, zero when saving

59      1       0FF if MGT Rom paged

60      1       0FF if Multiface Rom paged. Should always be 0.

61      1       0FF if 0-8191 is RAM

62      1       0FF if 8192-16383 is RAM

63      10      5x keyboard mappings for user defined joystick

(Default values: 0x0103,0x0203,0x0403,0x0803,0x1003)

73      10      5x ascii word: keys corresponding to mappings above

(Default values: 0x0031,0x0032,0x0033,0x0034,0x0035)

83      1       MGT type: 0=Disciple+Epson,1=Discipls+HP,16=Plus D

84      1       Disciple inhibit button status: 0=out, 0ff=in

85      1       Disciple inhibit flag: 0=rom pageable, 0ff=not

The value of the word at position 30 is 23 for version 2.01 files, and

54 for version 3.0x files.  The starred fields are the ones that

constitute the version 2.01 header, and their interpretation has

remained unchanged except for offset 34 (Hardware mode):

Value:          Meaning in v2.01        Meaning in v3.0x

0               48k                     48k

1               48k + If.1              48k + If.1

2               SamRam                  SamRam

3               128k                    48k + M.G.T.

4               128k + If.1             128k

5               -                       128k + If.1

6               -                       128k + M.G.T.

The hi T state counter counts up modulo 4.  Just after the ULA

generates its once-in-every-20-ms interrupt, it is 3, and is increased

by one every 5 emulated milliseconds to take the values 0,1,2 and 3

again respectively.  In these 1/200s intervals, the low T state counter

counts down from 17471 to 0 inclusive (or 17726 to 0 in 128K modes),

which make a total of 69888 T states (70908 T states) per frame.

The 5 ascii words (high byte always 0) at 73-82 are the keys

corresponding to the joystick directions left, right, down, up, fire

respectively.  Shift, Symbol Shift, Enter and Space are denoted by

[,],/,\ respectively.  The ascii values are used only to display the

joystick keys; the information in the 5 keyboard mapping words

determine which key is actually pressed (and should correspond to the

ascii values).  The low byte is in the range 0-7 and determines the

keyboard row.  The high byte is a mask byte and determines the column.

Enter for example is stored as 0x0106 (row 6 and column 1) and 'g' as

0x1001 (row 1 and column 4).  The default values correspond to the Left

Sinclair Interface 2 joystick.

Byte 60 must be zero, because the contents of the Multiface RAM is not

saved in the snapshot file.  If the Multiface was paged when the

snapshot was saved, the emulated program will most probably crash when

loaded back.

Bytes 61 and 62 are a function of the other flags, such as byte 34, 59,

60 and 83.

Hereafter a number of memory blocks follow, each containing the

compressed data of a 16K block.  The compression is according to the

old scheme, except for the end-marker, which is now absent.  The

structure of a memory block is:

Offset  Length  Description

0       2       Length of data (without this 3-byte header)

2       1       Page number of block

3       [0]     Compressed data

The pages are numbered, depending on the hardware mode, in the

following way:

Page    In '48 mode     In '128 mode    In SamRam mode

0      48K rom         rom (basic)     48K rom

1      Interface I, Disciple or Plus D rom, according to setting

2      -               rom (reset)     samram rom (basic)

3      -               page 0          samram rom (monitor,..)

4      8000-bfff       page 1          Normal 8000-bfff

5      c000-ffff       page 2          Normal c000-ffff

6      -               page 3          Shadow 8000-bfff

7      -               page 4          Shadow c000-ffff

8      4000-7fff       page 5          4000-7fff

9      -               page 6          -

10      -               page 7          -

11      Multiface rom   Multiface rom   -

In 48K mode, pages 4,5 and 8 are saved.  In SamRam mode, pages 4 to 8

are saved.  In '128 mode, all pages from 3 to 10 are saved.  This

version saves the pages in numerical order.  There is no end marker.

This concludes the specification of .Z80 files.  .SLT files (which

stands for 'super level loader trap files') are like .Z80 files except

that after the ordinary data another section follows, containing things

like level data (previously stored in .DAT files, and giving .SLT files

their name) or loading screens.  The ".Z80"-file preceding the SLT must

conform the format of v2.01 or v3.0x files (long header).

Though it is agreed that .Z80 files will not contain SLT data, and

files with SLT data will always have extension .SLT, Z80 does in fact

not distinguish between the extensions.

The SLT format was cooked up by Damien Burke, James McKay and yours

truly.  It starts as an ordinary >= v2.01 .Z80 file.  Directly

following this comes a

Offset  Length  Description

0       6       Separator (0,0,0,'S','L','T')

Then a table follows, each entry describing a piece of data.  The

format of a single table entry is:

Offset  Length  Description

0       2       Data type: 1=level data, 3=loading screen

2       2       Id word: Level number (between 0 and 255 inclusive)

for type 1, border colour (between 0 and 7) for

type 3.

4       4       Length of data block in bytes (Note: long word)

Data types other than type 1 and 3 are not supported by Z80 v3.04, and

are ignored.  The table ends with an all-zero end marker:

Offset  Length  Description

0       8       End marker (all zeroes)

Finally, the data blocks follow.  The blocks are stored in the order in

which they appear in the table,  i.e. the offset of a particular data

block is the sum of the lengths of the blocks that precede it.  The

internal format of these blocks depend on the data type:

Type:   Format:

0       (no data)

1       Compressed data (ED ED xx yy scheme, see above) expanding

to anything up to 48K

3       Compressed data expanding to exactly 6912 bytes of data

See the end of section 5.8 for an explanation how to load these blocks

using the ED FB opcode, and error handling.

All words in the format have the least significant byte first.